CZT_BirdFramework.h

Written by David Swigger (5/19/2003)

Last Update (1/5/2004)

The CZT_BirdFramework.h file is meant to be used in tandem with the ztBirdControlPack OCX controls.  The CZT_BirdFramework.h  consists of a namespace containing enumerations and constants that allow a developer to manipulate the OCX controls.

Usage:

To use the namespace in a project already containing the OCX controls include the header and declare usage of the namespace (this can be done in your sources header or .cpp)

#include "CZT_BirdFramework.h"

using namespace ztBirdEngine;

All properties of the OCX controls are set using a combination of enum values and specific method calls.

When setting a property on the control two values are passed in, the first parameter is the enumeration of the property, the second is the value.  When retrieving a property only the enumeration is required.

Bird Control Values (general):

| Enumeration | ## Description | Uses |
| --- | --- | --- |
| vBirdID | The identication of a bird – like “birdctrl1” | string |
| vBirdTempFilePath | While recording temporary wave files are generated. This paramer sets the path where they will be recorded. | string |
| vBirdWaveFile | Path to a wave file that can be triggered to play by calling the TriggerSound method. | string |
| vBirdWaveFilePath | Where wave files are moved when they are within tolerence. | string |
| vBirdState | Allows a birds state to modified | ztBirdStates |
| vPeakThreshold | Threshold (frequency) at which sound should be recognized | long |
| vPeakRecordDuration | The minimum amount of time to record (in milliseconds) once the threshold has been breached. | long |
| vRecordingCutoffMS | The maximum amount of time (in milliseconds) that a recorded wave file should contain. | long |
| vPeakComparator | Comparator to determine if recorded wave file is within tolerence. This value is compared to a peak count of a recording duration. | long |
| vMonitorDisplay | What is displayed when monitoring signal input | ztDisplays |
| vRecordingDisplay | What is displayed when recording signal input | ZtDisplays |
| vBirdName | Name used in display and to generate filenames | string |
| vAudioSrcDescription | Description of audio src (device and channel) | string |
| vAudioTrgDescription | Description of audio target (device and channel) | string |
| vBirdMasterID | ID of master for bird in slave mode (see states) | string |
| vDirectRecord | Used to trigger or stop recording when a bird is in DirectRecord mode (see states) | ztDirectRecordFlags |
| vPeakHP_Filter | If set to non-zero will enable a high-pass filter in the peak detection algorithms. The filtered data will only be used for comparison and display – the raw PCM data is still recorded. | long |
| vPeakGain | The peak gain is a float from .5f through xx.xxxxxx which will scale the peaks (after high pass). The scale is used in display and calculation of peaks (IE: it does not affect recorded wavform data) | float |

Bird Control Values (ztDisplays):

Each bird can have a different display while the engine is active.  The display type enumeration

Is located in the header file, to change the display SetLongValue is used in combination with one of the display enumerations.  In the header file there is also a constant string array named zt_DisplayType_Description which allows a developer to build a selectable list of valid display types.  If (for example) a developer wanted to iterate all the available display type they could write something like this:

For(int I=0;I<ztDisplayType_Total;I++)

{

AddToMyList(I,zt_DisplayType_Description[I]);

}

Bird Control States/Modes (ztBirdStates):

Each bird can be in a couple different states.  A bird state is set using the SetLongValue and the vBirdState enumeration.  The parameter passed in can be a combination ( or’ed together) of the following:

| ztbs_Unitialized | The bird is not active or monitoring (idle) |
| --- | --- |
| ztbs_Active | The bird is active and should process data |
| ztbs_MonitoringOnly | The bird is not active but should passively process data just like it would were it active (IE: no wave files are produced) |
| ztbs_DirectRecordMode | The bird should be put into direct record mode, meaning it will only record when the vDirectRecord long value is sent in with either the start or stop DirectRecord enumeration. |
| ztbs_SlaveMode | The bird will be controlled by another bird. The master bird is set using the vBirdMasterID enumeration. |

Bird Engine Values :

| Enumeration | Description | Uses |
| --- | --- | --- |
| vEngineState | State of the bird engine (read only) | ztEngineStates |
| vBirdEngineVersion | Version of the ocx engine (read only) | string |

Bird Engine States (ztEngineStates):

| ztes_Idle | The engine is stopped and idle |
| --- | --- |
| ztes_Running | The engine is currently running |
| zted_Stopping | The engine is in the process of stopping |